Rottendog Amusements

MPU327

Combination Williams System 3, 4, & 6 MPU and Driver Board

Installation Instructions:

- With power OFF, remove the original MPU and Driver Board from the game

- Remove the two screws which hold the center support for the old MPU and Driver Board

- Remove the center support bracket.

- Fasten the MPU327 board where the original Driver Board was located with 4 corner screws

- Remove the cables from the 3 cable holders that run across the top left of the back box

Run Self Test

  - Attach ONLY the 1J2 connector onto the top left of the MPU327

  - Set the DIP Switch DS3 to all 1’s (push them all downward)

  - Remove Battery from Battery Holder BT1

  - Turn power to the Machine to ON

  - Verify the 2 LED’s (LED1 and LED2) flash ON and OFF at about 1 second intervals

  - Press SW1 (Diagnostics Switch) briefly while both LED1 and LED2 are OFF and release

  - After about a 5sec pause, verify LED1 goes ON, then goes back OFF

  - Verify LED2 goes ON, then goes back OFF

  - Then Verify LED1 and LED2 flash ON and OFF as per line e.

  - Turn Power to the Machine to OFF

If any step above fails to function as described, the MPU327 board is not working properly!

Note:  This diagnostic code credit to Leon at http://www.flipper-pinball-fan.be/

- Set DIP switch DS3 to your specific game as shown in silk screen (leftmost 2 switches not used but should be set to “00”

- Attach all of the connectors across the top of the MPU326 (1J3, 1J4, 1J5, 1J6, 1J7)

- Verify battery is still removed from MPU327

- Turn pinball machine to “ON”

- Verify LED’s display “Audits” mode in player 1 and Ball & Credit score

- Attach battery while machine is still ON (Writing on Battery should be visible)

- Turn pinball machine to “Off”

- Attach all remaining connectors (note 1J8 is only used for System 7 games)

- Turn pinball machine to “ON” and play your game

1/30/2018