STANDARD MICROSYSTEMS CORPORATION

SuperDisk

For Ethernet LAN Adapters

This software is licensed by SMC for use by its customers only.

Copyright (c) 1994 Standard Microsystems Corporation.

All rights reserved.

Version 2.4       17 November 1995

Compressed Driver File SMCG24.EXE

DISK INFORMATION

This SMC SuperDisk is designed to be used with SMC's EZStart Program,

which is on this SuperDisk.  EZStart configures and tests your adapter,

copies the files you need from this disk to your PC, and displays or

prints appropriate instruction files that are located on this disk.

SYSTEM REQUIREMENTS

To use EZStart, your system must meet the following minimum requirements:

o  MS-DOS or PC-DOS operating system, version 3.11 or later, which

is required to run the EZStart program

o  To take advantage of EZStart's graphics features, a VGA or better

monitor; however, EZStart can operate in text mode on any monitor

o  A minimum amount of available hard disk space (typically 50 KB)

for installing a single driver from the SuperDisk using EZStart

OR

At least 1.3 MB of available hard disk space if you intend to

extract all of the drivers from the SuperDisk

Note: Refer to the "Installing A Network Driver" section later in

this document or the User Manual for more information on

copying driver files from the SuperDisk.

EZDOSODI - For Automatic NetWare Client Installation

With a single keystroke, EZDOSODI configures the EtherEZ adapter and

your computer for use as a DOS client in a NetWare server environment

by installing one of two SMC DOS ODI drivers:

o  If your adapter is configured for the default I/O Mapped

Addressing Mode, EZDOSODI installs SMC's Turbo DOS ODI driver

(an enhanced performance driver), SMC8416.COM. This driver can

significantly improve your workstation's networking performance.

o  If your adapter is configured for Memory Mapped Addressing Mode,

EZDOSODI installs SMC's dual-mode DOS ODI driver, SMC8000.COM.

This driver works in either I/O Mapped or Memory Mapped Addressing

Mode. It supports the EtherEZ adapter as well as SMC's

EtherCard Elite16 Ultra adapter.

For more information about EZDOSODI, refer to the Quick and EZ Guide

provided with your adapter and to the DOS ODI driver installation

document in the DOS_ODI subdirectory (INSTALL.DOC).

NOTE: To install some other network driver, or if your environment

requires greater customization, use EZStart, as described

in the section titled, USING EZSTART.

Before running EZDOSODI:

Exit Windows

Install the adapter into an EISA or ISA bus slot

Load other device drivers for your system, but do not load any

other network drivers

Cable the adapter to the network

To run EZDOSODI:

Insert the SuperDisk in a drive, change to that drive, type

EZDOSODI, and press Enter. (If you know are using 802.3 frame

type, type EZDOSODI 802.3 and press Enter for a faster

installation.)

EZDOSODI will automatically:

Configure and test the adapter.

Edit your AUTOEXEC.BAT file and save the old version as

AUTOEXEC.00x.

Copy the driver files to C:\DOS_ODI.

Select an Ethernet frame type (802.2/802.3).

Load the appropriate DOS ODI driver.

Display a login prompt, allowing you to login to the

nearest server.

If any step cannot be completed successfully, EZDOSODI exits to

EZStart (described below), where you can continue the installation.

NOTE: For those environments where Windows for Workgroups is installed

and configured for networking capability in a Novell

environment, EZDOSODI is not recommended. (EZDOSODI loads Novell

ODI drivers and may conflict with Windows for Workgroups, which

commonly uses NDIS drivers.) For those environments that run

Windows for Workgroups and Novell concurrently, use the EZStart

utility to manually configure and test the adapter and to

install drivers.

USING EZStart

EZStart is an all-purpose utility program that lets you:

Automatically configure and test the adapter

Install the network driver

Install and configure PC Agent/SNMP software, and view

network management information it collects

In addition, EZStart makes it easy to perform optional installation

and diagnostic procedures:

Manually configure the adapter by selecting a value for

each configuration option on an individual basis

Test two or more adapters communicating with each other

over your actual network

View the configurations of all installed adapters

LOADING EZStart

EZStart can run from the diskette, or for faster operation it can be

copied onto a hard disk and run from there.

Note: To run EZStart from the hard disk, copy all the files from

the SuperDisk to your computer's hard disk.  However, make sure

that the compressed driver file, SMCG24.EXE, and EZSTART.EXE

reside in the same directory.

To load EZStart, from the DOS command line, type:

[path]ezstart<Enter>

To load EZStart in Text Mode (instead of Graphics Mode), type

[path]ezstart/T<Enter>

To load EZStart from Microsoft Windows, select the File menu in

Program Manager.  When the Run dialog box appears, type:

[path]ezstart

in the Command Line box, then select the OK button.

INSTALLING A NETWORK DRIVER

EZStart accesses network driver files through the compressed

driver file, SMCG24.EXE.  The file name, SMCG24.EXE, specifies

the family of adapters supported ("G" denotes the SMC EtherEZ

Adapter), and the version of the SuperDisk ("24" denotes 2.4).

EZStart copies the network driver and any other supporting files

from the compressed driver file to a location compatible with

your network operating system.  If your network operating system

requires an installation or options floppy diskette, or your PC

has a single floppy drive, you can build the installation or

options diskette on the SuperDisk by following these steps:

Make a backup copy of your original SuperDisk

Load EZStart

When prompted for the Destination Directory by the EZStart

Driver Installation window, specify the floppy drive which

contains the SuperDisk (e.g., a:\)

Note: If you prefer, you can extract all of the network drivers

from the SuperDisk by:

Changing your default directory to the floppy drive where

the SuperDisk resides

Typing SMCG24 <Destination Directory> at the DOS prompt

(where <Destination Directory> is the path where you want

the extracted SuperDisk contents to reside)

Note: There must be at least 1.3 MB of available hard disk space

SuperDisk CONTENTS

The following list describes some of the files contained in

SMCG24.EXE:

File                       Contents

README.DOC     This file.

!SMCxxxx.CFG   EISA configuration files:

!SMC8003.CFG - For EtherCard PLUS Elite Family

8-bit LAN Adapters (8003xx)

!SMC8013.CFG - For EtherCard PLUS Elite 16 Family

16-bit LAN Adapters (8013xx)

!SMC8216.CFG - For EtherCard Elite Ultra LAN

Adapters (8216xx)

!SMC8416.CFG - For EtherEZ Plug and Play

Adapters (8416xx)

Drivers for many LAN operating systems and software utilities are

also available through EZStart:

Utilities/     Contents

Drivers

DOS ODI        Open data-link interface driver to be used with

ODI protocol stacks.

EZSTART        Documentation for the EZStart Program.

EZDOSODI       Batch file that performs an EZStart macro for

automatic NetWare client installation.

MS LAN         DOS and OS/2 Microsoft LAN Manager drivers.

MANAGER

NDIS           DOS and OS/2 NDIS drivers to be used with NDIS-

compliant network operating systems and protocol

stacks. Subdirectories contain documentation and

.INI files for specific network operating systems.

NETWARE 286    Documentation for drivers for Novell NetWare 286.

NETWARE 386    Server driver for Novell NetWare 386 version 3.1.

Novell NetWare Client for Microsoft Windows NT.

NETWARE        Driver for Novell NetWare 386 version 4.0x.

386 4.0x

OS2 ODI        Driver for OS/2 nodes in Novell NetWare

environments.

PCAGENT        Documentation and driver for SMC's PC Agent/SNMP

network-management utility.

WINDOWS NT     Driver for Microsoft Windows NT.

WINDOWS for     Driver for Microsoft Windows for Workgroups 3.11.

WORKGROUPS

WINDOWS 95     Driver for Microsoft Windows 95.

In addition to the driver or utility program itself, each

driver has the following associated files:

o  An INSTALL.DOC or other .DOC file that explains how to install

and use the software; and

o  A RELEASE.DOC file that states the current version, release date,

and any improvements made in this version of the software.

SUPPORTED ADAPTERS

The software on this SuperDisk operates with the following SMC

or Western Digital (WD) ISA and Micro Channel bus LAN adapters:

Ethernet Coax:

EtherCard Elite 16 Ultra (SMC 8216)

EtherEZ Plug and Play (SMC8416B)

Ethernet Twisted Pair:

EtherCard Elite 16T Ultra (SMC 8216T)

EtherEZ Plug and Play (SMC8416T)

Ethernet Combo:

EtherCard Elite 16C Ultra Combo (SMC 8216C)

EtherEZ Plug and Play (SMC8416BTA)

Ethernet Duo:

EtherEZ Adapter (SMC 8416BT)

BOOT ROMs FOR SMC ETHERNET ADAPTERS

Model             Adapters                  Protocols Supported

Number            Supported

SMC-EROM    Elite Ultra (8216xx)         Native NetWare Protocol

SMC EtherEZ (8416xx)   and RPL (Remote

Program Load)

Boot Protocol

ROMNOV F001       EtherCard PLUS Elite      Native NetWare

(8003xx and 8013xx)

ROMRPL            EtherCard PLUS Elite      RPL

(8003xx and 8013xx)

TECHNICAL SUPPORT

For technical support, first contact your place of purchase. If you

need additional assistance, call our Technical Support departments:

New York (8:30 am - 6:00 pm Eastern Time)

(800) 992-4762  (from U.S.A. and Canada)

(516) 435-6250

(516) 434-9314  (Fax)

California (7:00 am - 5:00 pm Pacific Time)

(800) 992-4762  (from U.S.A. and Canada)

(714) 707-2491  (Fax)

SMC offers Bulletin Board Services (BBS) from which you can obtain

the latest information on adapters and drivers for SMC products via

modem:

California    (714) 707-2481

9600 baud, 8 data bits, no parity, 1 stop bit

New York      (516) 434-3162

9600 baud, 8 data bits, no parity, 1 stop bit

Germany       49 (89) 928806-50

2400 (V.22bis) or 4800 (V.32bis) baud, 8 data

bits, no parity, 1 stop bit

France        33 (1) 39.73.57.00

1200 to 14K baud, 8 data bits, no parity, 1 stop bit

United        44 (0) 932 350681

Kingdom       2400 or 19.2 (HST only), 8 data bits, no parity,

1 stop bit

INTERNET Support:  SMC Technical Support can also be obtained through

the INTERNET. Our address is:  techsupt@ccmail.west.smc.com

CompuServe Support:  For information and drivers for SMC network

products, you can also access the SMC Forum through CompuServe. Log

into CompuServe, and at the "!" prompt type "GO SMC".

USING MEMORY MANAGERS

When operating in memory-mapped mode (for drivers created for the

EtherCard Elite16 adapter, or drivers created by third party developers

that do not support all of the EtherEZ features) and using a

memory-management utility, such as EMM386 or QEMM, you must exclude

an 8 KB RAM window for use by the adapter in your CONFIG.SYS file.

Then, before running EZStart, reboot your computer.

For example:

DEVICE=C:\DOS\EMM386.EXE X=D000-D1FF

For adapters with Boot ROMs, you must exclude an additional 8 KB,

for a total excluded memory space of 16 KB.

For example:

DEVICE=C:\DOS\EMM386.EXE X=D000-D3FF

Microsoft Windows 3.x and Windows for Workgroups users must also

exclude the RAM Window from the "386 Enhanced" area of the

SYSTEM.INI file in the WINDOWS subdirectory.  For example:

[386Enh]

EMMEXCLUDE=D000-D3FF

If you use a memory manager other than EMM386, refer to the manual

that came with it for information on how to exclude memory.

EMM386 v4.48 or Earlier

If you are using EMM386 v4.48 or earlier (provided with DOS v6.21

and earlier), upgrade to EMM386 v4.49 by obtaining DOS v6.22 or later,

or use a different memory manager. Otherwise, incompatibilities in

EMM386 may cause your computer to hang when a LAN driver attempts to

load. To determine your DOS version, type VER and press Enter at the

DOS prompt.

Memmaker and EMM386

Using the DOS v6.21 or v6.22 Memmaker program may cause system

reboots. Selecting Memmaker's Aggressive memory scanning option will

cause Memmaker to specify HIGHSCAN as an option parameter to EMM386

in the CONFIG.SYS file. The HIGHSCAN option may cause EMM386 to map

memory into the F000:0 BIOS space used by the system BIOS. In this

instance, EMM386 does not recognize that the BIOS may be using the

entire 64 KB range from F000:0 to F000:FFFF. To resolve this problem,

do not select the Aggressive memory scan option on Memmaker.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and EtherEZ,

EtherCard Elite 16 Ultra, EtherCard Elite 16T Ultra, EtherCard Elite

16C Ultra, EliteSeries, EZStart, and SuperDisk are trademarks of

Standard Microsystems Corporation.  Other product and company names

are registered trademarks or trademarks of their respective holders.