Williams System 3-7 Fuse Listing

Main fuse: All System 3 to System 7 games use a main fuse of 7.5 amp fast blow in the front of the cabinet (accessed through the coin door).

System 3 Fuses:

Power Supply Fuses:

    - F1 = Score display 90 volts AC, .25 amp Slow Blow.

    - F2 = Solenoids 28 volts DC, 2.5 amp Slow Blow.

    - F3 = Lamp matrix 18 volts DC, 8 amp Fast Blow.

    - F4 = GI fuse 6.3 volts, 20 amp Fast Blow, Hot Tip and Lucky Seven only. World Cup to Flash, this fuse is not used.

    - F5 = +5 volts DC logic, 4 amp Fast Blow.

Sound Board Fuses (except Hot Tip & Lucky Seven):

    - F1 = 9.3 volts AC, 4 amp Slow Blow (mislabeled 2 amp on some schematics).

Backbox Panel Fuses (located below power supply board):

    - Fuse Card = 28 volt flippers, 20 amp Fast Blow (not present on Hot Tip and Lucky Seven).

Playfield Fuse (located under playfield):

    - 28 volt flippers, 10 amp Fast blow, on games before Flash.

System 4 Fuses:

Power Supply Fuses:

    - F1 = Score display 90 volts AC, .25 amp Slow Blow.

    - F2 = Solenoids 28 volts DC, 2.5 amp Slow Blow.

    - F3 = Lamp matrix 18 volts DC, 8 amp Fast Blow.

    - F4 = Not used on games before Flash. Starting with Flash, F4 is the flipper fuse (10 amp Fast blow).

    - F5 = +5 volts DC logic, 4 amp Fast Blow.

Sound Board Fuses:

    - F1 = 9.3 volts AC, 4 amp Slow Blow.

    - F2 = 9.3 volts AC, 4 amp Slow Blow.

Backbox Panel Fuses (located below power supply board):

    - Fuse Card = 6.3 volt General Illumination, 20 amp Fast Blow.

Playfield Fuse (located under playfield):

    - 28 volt flippers, 10 amp Fast Blow (games before Flash only).

System 6 Fuses:

Power Supply Fuses:

    - F1 = Score display 90 volts AC, .25 amp Slow Blow.

    - F2 = Solenoids 28 volts DC, 2.5 amp Slow Blow.

    - F3 = Lamp matrix 18 volts DC, 8 amp Fast Blow.

    - F4 = Flippers 28 volts DC, 10 amp Slow Blow (GI fuse on shuffle alleys).

    - F5 = +5 volts DC logic, 4 amp Fast Blow.

Sound Board Fuses:

    - F1 = 9.3 volts AC, 4 amp Slow Blow.

    - F2 = 9.3 volts AC, 4 amp Slow Blow.

Backbox Panel Fuses (located below power supply board on a fuse card):

    - 6F1 Yellow Wires = General Illumination 6.3 volts AC, 20 amp Fast Blow.

    - 6F2 Gray Wires = Logic 9.3 volts AC supply, 4 amp Slow Blow.

    - 6F3 Gray Wires = Logic 9.3 volts AC supply, 4 amp Slow Blow.

No playfield fuses, as the fuse F4 on the power supply board is now used for the flippers.

System 7 Fuses.

Power Supply Fuses:

    - F1 = Score display 90 volts AC, .25 amp Slow Blow.

    - F2 = Solenoids 28 volts DC, 2.5 amp Slow Blow.

    - F3 = Lamp matrix 18 volts DC, 8 amp Fast Blow.

    - F4 = Flippers 28 volts DC, 10 amp (2 flippers) or 15 amp (3 or 4 flippers) Fast Blow (on shuffle alleys this is the GI fuse). This fuse is NOT used on Firepower2, LaserCue, Starlight.

    - F5 = 9.3 volts AC (input for +5 volts), 7 amp Slow Blow.

    - F6 = 9.3 volts AC (input for +5 volts), 7 amp Slow Blow.

    - F7 = General Illumination for pinballs, 6.3 volts AC, 20 amp Fast Blow.

Flipper Power Supply (Firepower2, LaserCue, Starlight ONLY):

    - F2 = 48 volts AC, 5 amp Slow Blow.

Sound Board Fuses:

    - F1 = 9.3 volts AC, 4 amp Slow Blow.

    - F2 = 9.3 volts AC, 4 amp Slow Blow.

Backbox Panel Fuses:

None, unless user added!

No playfield fuses, as the flipper fuse F4 is now on the power supply board or the flipper power supply board.